TASK: ESP BLE Provisioning

#### Description

This project demonstrates how to provision an ESP32 to a Wi-Fi network using Bluetooth Low Energy (BLE). It uses the ESP32’s BLE capability to allow users to connect to the device and input Wi-Fi credentials via a mobile app, making the Wi-Fi setup seamless and easy. Once connected, the device stores the Wi-Fi credentials in non-volatile storage, allowing it to reconnect automatically even after a power cycle. This is particularly useful in environments where the device may be deployed without direct access to update the Wi-Fi credentials.

The provisioning process leverages the Espressif Wi-Fi Provisioning Manager library to handle BLE interactions securely and efficiently. Users can use the Espressif Provisioning App on their smartphone to enter Wi-Fi credentials, which are then sent to the ESP32 over BLE. This code is compatible with the Arduino IDE and requires the WiFiProv library, which can be installed through the Arduino Library Manager.

#### Required Libraries

- WiFiProv: Manages BLE provisioning

- WiFi: Handles Wi-Fi connections

### Problem Solved by This Task

Setting up Wi-Fi connectivity on IoT devices can be challenging, particularly if the device is installed in hard-to-reach places or lacks an accessible interface. BLE provisioning solves this problem by allowing the device to advertise itself over BLE so users can connect with a mobile device to input Wi-Fi credentials. This is:

- Efficient: Reduces the need for physical setup interfaces like screens or keyboards.

- User-Friendly: BLE is a common feature on smartphones, making the setup straightforward.

- Secure: Uses Proof of Possession (PoP) to prevent unauthorized access during provisioning.

#### Code Overview

#include "WiFiProv.h"

#include "WiFi.h"

const char *pop = "abcd1234";

const char *service_name = "PROV_123";

const char *service_key = NULL;

bool reset_provisioned = true;

void SysProvEvent(arduino_event_t *sys_event) {

switch (sys_event->event_id) {

case ARDUINO_EVENT_WIFI_STA_GOT_IP:

Serial.print("\nConnected! IP address: ");

Serial.println(IPAddress(sys_event->event_info.got_ip.ip_info.ip.addr));

break;

case ARDUINO_EVENT_WIFI_STA_DISCONNECTED:

Serial.println("\nDisconnected. Reconnecting...");

break;

case ARDUINO_EVENT_PROV_START:

Serial.println("\nProvisioning started\nEnter Wi-Fi credentials using the smartphone app.");

break;

case ARDUINO_EVENT_PROV_CRED_RECV: {

Serial.println("\nReceived Wi-Fi credentials:");

Serial.print("\tSSID: ");

Serial.println((const char *)sys_event->event_info.prov_cred_recv.ssid);

Serial.print("\tPassword: ");

Serial.println((const char *)sys_event->event_info.prov_cred_recv.password);

break;

}

case ARDUINO_EVENT_PROV_CRED_FAIL: {

Serial.println("\nProvisioning failed. Try resetting to factory settings.");

if (sys_event->event_info.prov_fail_reason == WIFI_PROV_STA_AUTH_ERROR) {

Serial.println("Incorrect Wi-Fi password.");

} else {

Serial.println("Wi-Fi AP not found. Try erasing NVS with 'nvs_flash_erase()' before beginProvision().");

}

break;

}

case ARDUINO_EVENT_PROV_CRED_SUCCESS:

Serial.println("\nProvisioning Successful! Device is now connected.");

break;

case ARDUINO_EVENT_PROV_END:

Serial.println("\nProvisioning ended.");

break;

default:

break;

}

}

void setup() {

Serial.begin(115200);

WiFi.onEvent(SysProvEvent);

Serial.println("Starting BLE Provisioning...");

uint8_t uuid[16] = {

0xb4, 0xdf, 0x5a, 0x1c, 0x3f, 0x6b, 0xf4, 0xbf,

0xea, 0x4a, 0x82, 0x03, 0x04, 0x90, 0x1a, 0x02

};

WiFiProv.beginProvision(WIFI_PROV_SCHEME_BLE,

WIFI_PROV_SCHEME_HANDLER_FREE_BTDM,

WIFI_PROV_SECURITY_1,

pop,

service_name,

service_key,

uuid,

reset_provisioned);

Serial.println("BLE QR Code for provisioning:");

WiFiProv.printQR(service_name, pop, "ble");

}

void loop() {

}

Explanation of the working principle of code

- Provisioning Parameters:

  - pop: Proof of Possession (PoP) is a password required to authenticate during the provisioning process.

  - service_name: Sets the Bluetooth device name, visible to the app.

  - service_key: Password for SoftAP (not required for BLE provisioning).

  - reset_provisioned: Erases previous provisioning data to ensure new credentials are used.

- Event Handler (SysProvEvent):

  - Handles various events during the provisioning process, such as Wi-Fi connection, credential receipt, provisioning failure, and successful connection.

  - Outputs relevant logs to the Serial Monitor, which can be helpful for debugging.

- BLE Provisioning Setup:

  - The UUID used allows the app to recognize the device.

  - beginProvision() initializes provisioning with BLE, using security mode 1 and a predefined PoP.

- QR Code for Provisioning:

  - A QR code with the device name and PoP is generated, allowing users to quickly connect to the device via the provisioning app by scanning the code.

### Setup Instructions

- Install Dependencies:

  - Ensure the WiFiProv library is installed in the Arduino IDE.

  - Install the Espressif Provisioning App on your smartphone to complete the provisioning.

- Upload the Code:

  - Select the appropriate partition scheme (e.g., “No OTA (2MB APP/2MB SPIFFS)”) in Arduino IDE under Tools > Partition Scheme.

- Provisioning:

  - Open the Serial Monitor at 115200 baud rate to observe the logs.

  - Scan for your device in the app, enter the Wi-Fi credentials, and initiate provisioning.

- Monitor Output:

  - Logs in the Serial Monitor will indicate the provisioning progress, any connection errors, or successful provisioning events.

### Troubleshooting

- Provisioning Fails:

  - If provisioning fails, check the logs for errors.

  - You may need to call nvs_flash_erase() to clear previously saved data if you notice repeated failures due to incorrect credentials or connection issues.

- Device Not Visible in App:

  - Verify the ESP32 has BLE enabled and ensure your smartphone's Bluetooth is active.

  - Ensure the BLE UUID matches the format expected by the provisioning app.

Instructions for Uploading the Code

This code is memory-intensive, which may prevent it from flashing with default settings on some ESP32 boards. If you encounter a “Sketch too big” error during the upload, you’ll need to modify the partition scheme in the Arduino IDE:

1. Go to **Tools > Partition Scheme**.

2. Select a scheme that has at least **1.4MB of available space for the APP**. For example, try “**Huge APP (3MB No OTA/1MB SPIFFS)**.”

If the **Partition Scheme** option is not visible under the **Tools** menu, switch to a different board model such as the **ESP32 Wrover Module**, which supports larger partition sizes.

Testing the Code

To ensure the ESP32 Wi-Fi provisioning works correctly, follow these steps:

1. **Upload the Code**: Once you have uploaded the code to the ESP32, open the **Serial Monitor** in the Arduino IDE, setting the baud rate to **115200**.

2. **Start the Program**: Press the **RST (Reset) button** on the ESP32 to initiate the code execution.

**Note**: You may encounter a PSRAM error on the Serial Monitor. This error is safe to ignore, as it does not impact the Wi-Fi provisioning process.

3. **Observe Serial Monitor Logs**: In the Serial Monitor, you should see output messages confirming that the BLE provisioning process has started. The logs will resemble:

```

Begin Provisioning using BLE

Provisioning started

Provide Wi-Fi credentials using the smartphone app

```

Connecting with the Espressif Wi-Fi Provisioning App

4. **Launch the Espressif Wi-Fi Provisioning App**: Open the Espressif Wi-Fi Provisioning App on your smartphone.

5. **Provision Device**:

- In the app, select **Provision Device**.

- The app will display an option to **Scan QR Code**. If this option does not work, select **I don’t have a QR code**.

6. **Select ESP32 Device**:

- A list of available Bluetooth devices with the prefix “**PROV_**” will appear. In this case, the ESP32 device is configured to appear as **“PROV_123”**. Select this device to proceed.

7. **Enter Proof of Possession (PoP)**:

- When prompted, enter the Proof of Possession (PoP) value defined in the code. For this setup, the default PoP is **abcd1234**. Make sure this value matches the one in the code exactly.

8. **Select Wi-Fi Network**:

- After successful authentication, a list of nearby Wi-Fi networks will appear.

- If your network does not appear, you can manually add it by selecting **Join Other Network**.

9. **Enter Wi-Fi Credentials**:

- Enter the password for your Wi-Fi network and tap **Connect**.

- After a few seconds, the Wi-Fi credentials will be sent to the ESP32, allowing it to connect to the network.

10. **Confirm Connection**:

- In the **Serial Monitor**, you should see a confirmation message that the ESP32 has received the Wi-Fi credentials and has connected to the network successfully.

- The ESP32 will display its assigned **IP address** on the network.

Wi-Fi provisioning is now successfully implemented on the ESP32, allowing it to connect seamlessly to a network using the Espressif provisioning app.

### Conclusion

This project efficiently provisions an ESP32 to a Wi-Fi network using BLE, providing a simple, user-friendly solution for Wi-Fi configuration in IoT applications. The code is designed to be flexible and can be expanded for use in larger systems requiring Wi-Fi-based device control and monitoring.